# Chapter 2:   Fundamentals of Electrical Circuits – Instructor Notes

Chapter 2 develops the foundations for the first part of the book. The first four sections provide the basic definitions (Section 2.1) and cover Kirchoff’s Laws and the passive sign convention (Sections 2.2, 2.3 And 2.4); A special feature, Focus on Problem Solving: The Passive Sign Convention (p. 42) and two examples illustrate this very important topic.  A second feature that recurs throughout the first five chapters is presented in the form of sidebars. Make The Connection: Hydraulic Analog of a Voltage Source (p. 48), Make The Connection: Hydraulic Analog of Current Sources (p. 49), and Make The Connection: Hydraulic Analog of Electrical Resistance (p. 51) present analogies between electrical and other physical domains.

Sections 2.5 and 2.6 introduce the i-v characteristic, resistance and Ohm’s law, potentiometers, and power dissipation in resistors.  Table 2.1 on p. 51 lists the resistance of copper wire for various gauges and Tables 2.2 and 2.3 on p. 53 summarize the resistivity of common materials and standard resistor values. Section 2.7 discusses resistors in series and voltage division while Section 2.8 discusses resistors in parallel and current division.  Section 2.9 provides a thorough introduction to the important topic of equivalent resistance between two nodes, including handy calculation and approximation tips and the Wye-Delta Transformation. Selected applications of these topics are found immediately following Section 2.8, namely Focus on Measurements: Resistive throttle position sensor (pp. 66-67), Focus on Measurements: Resistance strain gauges (p. 68), and Focus on Measurements: The Wheatstone bridge and force measurements (pp. 69-70).  These topics are intended to motivate students, particularly non-electrical engineering students, through practical applications. The Wheatstone bridge material can serve as an introduction to a laboratory experiment on strain gauges and the measurement of force.

Sections 2.10 and 2.11 introduce some basic but important concepts related to ideal and non-ideal independent sources and measuring instruments. This material can motivate a number of practical lab exercises.

Although the material in Chapter 2 is quite basic, it is possible to give an applied flavor to the subject matter by emphasizing a few selected topics in the examples presented in class.  In particular, a lecture could be devoted to resistance devices, such as the resistive displacement transducer. The instructor wishing to gain a more in-depth understanding of resistance strain gauges will find a detailed analysis in the references.

The homework problems include a variety of practical examples, with emphasis on instrumentation.  Problem 2.45 illustrates analysis related to fuses; problem 2.43 relates to wire gauges; problem 2.70 discusses the thermistor; problem 2.71 discusses moving coil meters; problems 2.79 and 2.80 illustrate calculations related to strain gauge bridges; and a variety of problems related to practical measuring devices are also included in the last section.  The 6th Edition of this book includes new problems; some of the 5th Edition problems were removed or replaced, increasing the end-of-chapter problem count from 66 to 80.

It has been the authors’ experience that providing students with an early introduction to practical applications of electrical engineering to their own disciplines can increase the interest level in the course significantly.

### Learning Objectives for Chapter 2

- Identify the principal features of electric circuits or networks: nodes, loops,meshes, and branches.

- Apply Kirchhoff's laws to simple electric circuits.

- Apply the passive sign convention to compute the power consumed or supplied by circuit elements.

- Identify sources and resistors and their i-v characteristics.

- Apply Ohm's law and voltage and current division to calculate unknown voltages and currents in simple series, parallel, and series-parallel circuits.

- Correctly redraw a resistive network, as necessary, and compute the equivalent resistance between two nodes.

- Understand the impact of internal resistance in practical models of voltage and current sources as well as of voltmeters, ammeters, and wattmeters.

- Apply the source-load perspective to find graphical solutions to circuit problems.

- Identify the principal elements of electrical circuits: nodes, loops, meshes, branches, and voltage and current sources.

- Apply Kirchhoff’s Laws to simple electrical circuits and derive the basic circuit equations.

- Apply the passive sign convention and compute power dissipated by circuit elements.

- Apply the voltage and current divider laws to calculate unknown variables in simple series, parallel and series-parallel circuits.

- Understand the rules for connecting electrical measuring instruments to electrical circuits for the measurement of voltage, current, and power.

### Learning Objectives for Chapter 2

- Identify the principal elements of electrical circuits: nodes,  meshes, branches, and voltage and current sources.

- Apply Kirchhoff’s Laws to simple electrical circuits and derive the basic circuit equations.

- Apply the passie sign convention and compute power dissipated by circuit elements.

- Apply the voltage and current divider laws to calculate unknown variables in simple series, parallel and series-parallel circuits.

- Understand the rules for connecting electrical measuring instruments to electrical circuits for the measurement of voltage, current, and power.

Section 2.1: Definitions

# Problem 2.1

A free electron has an initial potential energy per unit charge (voltage) of 17 kJ/C and a velocity of 93 Mm/s.  Later, its potential energy per unit charge is 6 kJ/C. Determine the change in velocity of the electron.

## Solution:

### Known quantities:

Initial Coulombic potential energy, ; initial velocity,; final Coulombic potential energy, .

### Find:

The change in velocity of the electron.

### Assumptions:

### Analysis:

Using the first law of thermodynamics, we obtain the final velocity of the electron:

Heat is not applicable to a single particle. W=0 since no external forces are applied.

# Problem 2.2

The units for voltage, current, and resistance are the volt (V), the ampere (A), and the ohm (), respectively. Express each unit in fundamental MKS units.

## Solution:

### Known quantities:

MKSQ units.

### Find:

Equivalent units of volt, ampere, and ohm.

### Analysis:

# Problem 2.3

A particular fully charged battery can deliver 2.7 · 106 coulombs of charge.

## Solution:

### Known quantities:

qBattery = 2.7 · 106 C.

### Find:

- The capacity of the battery in ampere-hours

- The number of electrons that can be delivered.

### Analysis:

- There are 3600 seconds in one hour. Amperage is defined as 1 Coulomb per second and is directly proportional to ampere-hours.

2.7∙106C∙1hr3600s=750 AH

- The charge of a single electron is -1.602·10-19 C. The negative sign is negligible. Simple division gives the solution:

2.7∙106C1.602∙10-19C1 electron=1.685∙1025 electrons

# Problem 2.4

The charge cycle shown in Figure P2.4 is an example of a three-rate charge. The current is held constant at 30 mA for 6 h. Then it is switched to 20 mA for the next 3 h.

## Solution:

### Known quantities:

See Figure P2.4

### Find:

- The total charge transferred to the battery.

- The energy transferred to the battery.

### Analysis:

- Current is equal to CoulombsSecond, therefore given the current and a duration of that current, the transferred charge can be calculated by the following equation:

A∙t=C

The two durations should be calculated independently and then added together.

0.030A∙21600s=648C

0.020A∙10800s=216C

648C+216C=864C

- P=V·I, therefore, the total energy can be calculated as sum of the products of the average voltage, the average current, and the interval length over all three intervals. Note the conversion from hours to seconds.

U=V*I*t= (0.55*0.030*3600*3)+(0.9*0.030*3600*3)+(1.1*0.020*3600*3)=707.4J

# Problem 2.5

Batteries (e.g., lead-acid batteries) store chemical energy and convert it to electric energy on demand. Batteries do not store electric charge or charge carriers. Charge carriers (electrons) enter one terminal of the battery, acquire electrical potential energy, and exit from the other terminal at a lower voltage. Remember the electron has a negative charge! It is convenient to think of positive carriers flowing in the opposite direction, that is, conventional current, and exiting at a higher voltage. (Benjamin Franklin caused this mess!) For a battery with a rated voltage = 12 V and a rated capacity = 350 A-h, determine:

a. The rated chemical energy stored in the battery.

b. The total charge that can be supplied at the rated

## Solution:

### Known quantities:

Rated voltage of the battery; rated capacity of the battery.

### Find:

- The rated chemical energy stored in the battery

- The total charge that can be supplied at the rated voltage.

### Analysis:

a)

As the battery discharges, the voltage will decrease below the rated voltage. The remaining chemical energy stored in the battery is less useful or not useful.

b) is the total charge passing through the battery and gaining 12 J/C of electrical energy.

# Problem 2.6

What determines:

a. The current through an ideal voltage source?

b. The voltage across an ideal current source?

## Solution:

### Known quantities:

Resistance of external circuit.

### Find:

- Current through an ideal voltage source

- Voltage across an ideal current source.

### Assumptions:

Ideal voltage and current sources.

### Analysis:

a) An ideal voltage source produces a constant voltage at or below its rated current. Current is determined by the power required by the external circuit (modeled as R).

b) An ideal current source produces a constant current at or below its rated voltage. Voltage is determined by the power demanded by the external circuit (modeled as R).

A real source will overheat and, perhaps, burn up if its rated power is exceeded.

Problem 2.7

An automotive battery is rated at 120 A-h. This means that under certain test conditions it can output

1 A at 12 V for 120 h (under other test conditions, the battery may have other ratings).

a. How much total energy is stored in the battery?

b. If the headlights are left on overnight (8 h), how much energy will still be stored in the battery in the

morning? (Assume a 150-W total power rating for both headlights together.)

## Solution:

### Known quantities:

Rated discharge current of the battery; rated voltage of the battery; rated discharge time of the battery.

### Find:

- Energy stored in the battery when fully recharging

- Energy stored in the battery after discharging

### Analysis:

a)

b)  Assume that 150 W is the combined power rating of both lights; then,

Problem 2.8

A car battery kept in storage in the basement needs recharging. If the voltage and the current provided bythe charger during a charge cycle are shown in FigureP2.8,

a. Find the total charge transferred to the battery.

b. Find the total energy transferred to the battery.

## Solution:

### Known quantities:

Recharging current and recharging voltage

### Find:

- Total transferred charge

- Total transferred energy

### Analysis:

a)

b)  so

where

Therefore,

Problem 2.9

Suppose the current through a wire is given

by the curve shown in Figure P2.9.

a. Find the amount of charge, q, that flows through

the wire between t1 = 0 and t2 = 1 s.

b. Repeat part a for t2 = 2, 3, 4, 5, 6, 7, 8, 9, and 10 s.

c. Sketch  q(t) for 0 ≤ t ≤ 10 s.

## Solution:

### Known quantities:

Current-time curve

### Find:

- Amount of charge during 1st second

- Amount of charge for 2 to 10 seconds

- Sketch charge-time curve

### Analysis:

a)

b) The charge transferred from  to  is the same as from  to.

The charge transferred from  to  is the same in magnitude and opposite in direction to that from  to.

# Problem 2.10

The charge cycle shown in Figure P2.10 is an example of a two-rate charge. The current is held constant at 70 mA for 1 h. Then it is switched to 60 mA for the next 1 h.

## Solution:

### Known quantities:

See Figure P2.10

### Find:

- The total charge transferred to the battery.

- The total energy transferred to the battery.

### Analysis:

- Current is equal to CoulombsSecond, therefore given the current and a duration of that current, the transferred charge can be calculated by the following equation:

A∙t=C

The two durations should be calculated independently and then added together.

0.070A∙3600s=252C

0.060A∙3600s=216C

648C+216C=468C

- P=V·I, therefore, an equation for power can be found by multiplying the two graphs together.

First separate the voltage graph into three equations:

0 h  1 h : V=5+et5194.8V

1 h  2 h : V=6-4e1-1+4e2-e1*etV

Next, multiply the first equation by 0.07A and the second by 0.06A.

0 h  1 h : P=0.35+0.07et5194.8

1 h  2 h : P=0.066-4e1h-1+0.064e2h-e1h*etV

Finally, since Energy is equal to the integral of power, take the integral of each of the equations for their specified times and add them together.

0 h  1 h : E=0.35t+363.64et5194.803600= 1623.53 J

1 h  2 h :  E=0.36t+2.88*10-3128*2.72t36007200 = 1296.24 J

ETotal=2919.77 J

Problem 2.11

The charging scheme used in Figure P2.11 is an example of a constant-current charge cycle. The charger voltage is controlled such that the current into the battery is held constant at 40 mA, as shown in Figure P2.11. The battery is charged for 6 h. Find:

a. The total charge delivered to the battery.

b. The energy transferred to the battery during the charging cycle.

Hint: Recall that the energy, w, is the integral of power, or P = dw/dt.

## Solution:

### Known quantities:

Current-time curve and voltage-time curve of battery recharging

### Find:

- Total transferred charge

- Total transferred energy

### Analysis:

a)

b)  so

Problem 2.12

The charging scheme used in Figure P2.12 is called

a tapered-current charge cycle. The current starts at

the highest level and then decreases with time for the

entire charge cycle, as shown. The battery is charged

for 12 h. Find:

a. The total charge delivered to the battery.

b. The energy transferred to the battery during the

charging cycle.

Hint: Recall that the energy, w, is the integral of power, or

P = dw/dt.

## Solution:

### Known quantities:

Current-time curve and voltage-time curve of battery recharging

### Find:

- Total transferred charge

- Total transferred energy

### Analysis:

a)

b) so

Problem 2.13

Use KCL to determine the unknown currents in the circuit of Figure P2.13. Assume i0 = 2 A and i2 =−7A.

## Solution:

### Known quantities:

io = 2 A,  i2 = -7 A

### Find:

- i1

- i3

### Analysis:

- Use KCL at the node between Ro ,R1, and R2.

i0-i1+i2=0

i1=i0+i2

i1=-5A

- Use KCL at the node between R2 ,R3, and the current source.

6A+i3-i2=0

i3=i2-6A

i3=-13A

Problem 2.14

Use KCL to find the currents i1 and i2 in Figure P2.14. Assume that ia = 3 A, ib = −2 A, ic = 1 A, id = 6 A and ie = −4 A.

## Solution:

### Known quantities:

ia = 3 A,  ib = -2 A, ic = 1 A, id = 6 A, ie = -4 A

### Find:

- i1

- i2

### Analysis:

- Use KCL at Node A.

i1+ib-ia-ic=0

i1=ia-ib+ic

i1=6A

- Use KCL at Node B.

ie+id-i1-i2=0

i2=ie+id-i1

i2=-4A

Problem 2.15

Use KCL to find the currents i1, i2, and i3 in the circuit of Figure P2.15. Assume that ia = 2 mA, ib =7mA and ic =4mA.

## Solution:

### Known quantities:

ia = 2 mA,  ib = 7 mA, ic = 4 mA

### Find:

- i1

- i2

- i3

### Analysis:

- Use KCL at Node A.

ib-ia-i1=0

i1=ib-ia

i1=5mA

- Use KCL at Node C.

ic-i2-ib=0

i2=ic-ib

i2=-3mA

- Use KCL at Node D.

i3+ia-ic=0

i3=i2-ia

i3=2mA

Problem 2.16

Use KVL to find the voltages v1, v2, and v3 in Figure P2.16. Assume that va = 2 V, vb = 4 V, and vc = 5 V.

## Solution:

### Known quantities:

Va = 2 V,  Vb = 4 V,  Vc = 5 V

### Find:

- V1

- V2

- V3

### Analysis:

- Use KVL at the third loop.

V3-Vc=0

V3=Vc

V3=5V

- Use KVL at the second loop.

V2-V3-Vb=0

V2=V3+Vb

V2=9V

- Use KCL at the first loop.

Va-V1-V2=0

V1=Va-V2

V1=-7V

Problem 2.17

Use KCL to determine the currents i1, i2, i3, and i4 in the circuit of Figure P2.17. Assume that ia = −2 A, ib =6A, ic =1A and id =−4A.

## Solution:

### Known quantities:

ia = -2 A,  ib = 6 A,  ic = 1 A, id = -4 A

### Find:

- i1

- i2

- i3

- i4

### Analysis:

- Use KCL at Node A.

i1-ia-ic=0

i1=ia+ic

i1=-1A

- Use KCL at Node B.

i2-i1-ib=0

i2=i1+ib

i2=5A

- Use KCL at Node C.

i3-i2-id=0

i3=i2+id

i3=1A

- Use KCL at Node D.

ic+i4-i3=0

i4=i3-ic

i4=0A

# Problem 2.18

In the circuits of Figure P2.18, the current directions and voltage polarities are defined. Find the magnitude and sign of the indicated currents and voltages.

## Solution:

### Known quantities:

Circuit shown in Figure P2.18.

### Find:

Voltages and currents in every figure.

### Analysis:

(a)  Using :

(b)  The voltage across the 20  resistor is ; since the current flows from top to bottom, the polarity of this voltage is positive on top.  Then it follows that  and

(the negative sign follows from the direction of I2 in the drawing).

(c)  Since -0.5A pointing upward is the same current as 0.5A pointing downward, the voltage across the 30  resistor is

(positive on top); and ,

since is also the voltage across the 20  resistor.  Finally,

,and

# Problem 2.19

Find the power delivered by each source in Figure P2.19.

## Solution:

### Known quantities:

Circuits shown in Figure P2.19.

- Power delivered by the 3A Current Source

- Power delivered by the -9V Voltage Source

### Analysis:

- Follow the counterclockwise current:

P=(+3A)∙(+10V)

P=+30Wsupplied

- Follow the counterclockwise current:

P=(+5A)∙(-9V)

P=-45Wsupplied

Problem 2.20

Determine whether each element in Figure P2.20 is supplying or dissipating power, and how much.

## Solution:

### Known quantities:

Circuit shown in Figure P2.20.

### Find:

Determine power dissipated or supplied for each power source.

### Analysis:

Element A:

(dissipating)

Element B:

(dissipating)

Element C:

(supplying)

# Problem 2.21

In the circuit of Figure P2.21, find the power absorbed by the resistor R4 and the power delivered by the current source.

## Solution:

### Known quantities:

Circuit shown in Figure P2.21.

### Find:

- Power absorbed by R4

- Power delivered by the current source

### Analysis:

- Follow the counterclockwise current in the rightmost loop:

P=(2A)∙(-15V)

P=-30Wabsorbed

P=+30Wsupplied

This result violates Ohm’s law so that is one funky resistor!

- Use KVL at the leftmost loop to find V3:

10V-2V-1V-V3=0

V3=7V

Use KVL at the rightmost loop to find V5:

7V-15V-V5=0

V5=-8V

Remember that the power calculations are based upon the passive sign convention, in which current is directed from high to low potential. Positive power is power absorbed by an element. Since V5 is negative the current is directed from high to low potential such that power is absorbed by the current source, somewhat contrary to what may have been expected.

P = -+2A∙-8V=-2A-8V=+16W absorbed

Problem 2.22

For the circuit shown in Figure P2.22:

a. Determine whether each component is absorbing or delivering power.

b. Is conservation of power satisfied? Explain your answer.

## Solution:

### Known quantities:

Circuit shown in Figure P2.22.

### Find:

- Determine power absorbed or power delivered

- Testify power conservation

### Analysis:

By KCL, the current through element B is 5A, to the right.

By KVL, .

Therefore, the voltage across element A is

(positive at the top).

A supplies

B supplies

C absorbs

D absorbs

E absorbs

Total power supplied =

Total power absorbed =

Tot. power supplied = Tot. power absorbed

conservation of power is satisfied.

Problem 2.23

For the circuit shown in Figure P2.23, determine

the power absorbed by the 5 Ω resistor.

## Solution:

### Known quantities:

Circuit shown in Figure P2.23.

### Find:

Power absorbed by the resistance.

### Analysis:

The current flowing clockwise in the series circuit is

The voltage across the 5  resistor, positive on the left, is

Therefore,

Problem 2.24

Determine whether each element in Figure P2.24 is supplying or dissipating power, and how much.

## Solution:

### Known quantities:

Circuit shown in Figure P2.24.

### Find:

Determine power absorbed or power delivered and corresponding amount.

### Analysis:

If current direction is out of power source, then power source is supplying, otherwise it is absorbing.

A supplies

B absorbs

C supplies

D supplies , i.e absorbs

E absorbs

Problem 2.25

Determine whether each element in Figure P2.25 is supplying or dissipating power, and how much.

## Solution:

### Known quantities:

Circuit shown in Figure P2.25.

### Find:

Determine power absorbed or power delivered and corresponding amount.

### Analysis:

If current direction is out of power source, then power source is supplying, otherwise it is absorbing.

A absorbs

B supplies

D supplies

Since conservation of power is satisfied, Tot. power supplied = Tot. power absorbed

Total power supplied =

Cabsorbs

Problem 2.26

If an electric heater requires 23 A at 110 V, determine:

a. The power it dissipates as heat or other losses.

b. The energy dissipated by the heater in a 24-h period.

c. The cost of the energy if the power company charges at the rate 6 cents/kWh.

## Solution:

### Known quantities:

Current absorbed by the heater; voltage at which the current is supplied; cost of the energy.

### Find:

- Power consumption

- Energy dissipated in 24 hr.

- Cost of the Energy

### Assumptions:

The heater works for 24 hours continuously.

### Analysis:

a)

b)

c)

Problem 2.27

In the circuit shown in Figure P2.27, determine the terminal voltage vT of the source, the power absorbed by Ro, and the efficiency of the circuit. Efficiency is defined as the ratio of load power to source power.

## Solution:

### Known quantities:

Circuit shown in Figure P2.27 with voltage source,

vS =12V, internal resistance, RS =5k, and load, Ro =7k.

### Find:

The terminal voltage of the source; the power supplied to the circuit, the efficiency of the circuit.

### Assumptions:

Assume that the only loss is due to the internal resistance of the source.

### Analysis:

.

Problem 2.28

A 24-V automotive battery is connected to two headlights that are in parallel, similar to that shown in Figure 2.11. Each headlight is intended to be a 75-W load; however, one 100-W headlight is mistakenly installed. What is the resistance of each headlight? What is the total resistance seen by the battery?

## Solution:

### Known quantities:

Headlights connected in parallel to a 24-V automotive battery; power absorbed by each headlight.

### Find:

Resistance of each headlight; total resistance seen by the battery.

### Analysis:

Headlight no. 1:

Headlight no. 2:

The total resistance is given by the parallel combination:

or RTOTAL = 3.29

Problem 2.29

What is the equivalent resistance seen by the battery of Problem 2.28 if two 15-W taillights are added (in parallel) to the two 75-W headlights?

## Solution:

### Known quantities:

Headlights and 24-V automotive battery of problem 2.13 with 2 15-W taillights added in parallel; power absorbed by each headlight; power absorbed by each taillight.

### Find:

Equivalent resistance seen by the battery.

### Analysis:

The resistance corresponding to a 75-W headlight is:

For each 15-W tail light we compute the resistance:

Therefore, the total resistance is computed as:

or

Problem 2.30

For the circuit shown in Figure P2.30, determine the power absorbed by the variable resistor R, ranging from 0 to 30 . Plot the power absorption as a function of R. Assume that vS = 15 V, RS = 10 .

## Solution:

### Known quantities:

vs=15V, Rs=10 , and the circuit in Figure P2.30.

### Find: R

### Analysis:

Use Ohm’s law to find an equation for P as a function of R:

PR=VR*IR

The voltage across R is equal to the source voltage minus the voltage across Rs.

VR=15V-VRs

The voltage VRS is determined by voltage division.

VRS=15VRs(Rs+R)

Then, the power dissipated by R is

PR=15V-15V10Ω10Ω+R*15V10Ω+R

Plot:

PR

R

Problem 2.31

Refer to Figure P2.27 and assume that vS = 15 V and RS =100 .For iT =0, 10, 20, 30, 80, and  100 mA:

- Find the total power supplied by the ideal source.

- Find the power dissipated within the non-ideal source.

- How much power is supplied to the load resistor?

- Plot the terminal voltage vT and power supplied to  the load resistor as a function of terminal current iT.

## Solution:

### Known quantities:

vs=15V, Rs=100 Ohms, iT= 0, 10, 20, 30, 80, 100 mA.

Figure P2.27.

### Find:

- The total power supplied by the ideal source

- The power dissipated within the non-ideal source

- How much power is supplied to the load resistor

- Plot vT and power supplied to R0 as a function of iT.

### Analysis:

- The power supplied by the ideal source is equal to the current through the loop times the 15V of the supply. From current lowest to highest the power supplied would be:

0W          0.15W          0.3W          0.45W          1.2W          1.5W

- The power dissipated within the non-ideal source is the power dissipated by Rs which can be found using P=i2*r. From current lowest to highest the power dissipated would be:

0W          0.01W          0.04W          0.09W          0.64W          1W

- The power supplied to the load resistor is equal to the total power supplied minus the power dissipated by the non-ideal source. From current lowest to highest the power supplied would be:

0W          0.14W          0.26W          0.36W          0.56W          0.5W

- For the vT plot Ohm’s Law can be used to find the voltage drop across Rs which is equal to VT. For the power plot, the data from part c can be used directly.

Problem 2.32

In the circuit in Figure P2.32, assume v2 = vs/6 and the power delivered by the source is 150 mW. Find R, vs, v2, and i.

## Solution:

### Known quantities:

R1=8kΩ, R2=10kΩ, R3=12kΩ and the circuit in Figure P2.32.

### Find:

R, vs, v2, and i.

### Analysis:

Use ohm’s law to find vs and i:

v2=vs6=R2*i

Also:

vs*i=150mW

So:

i=2150mWR2*6=1.58 mA

Since we know i, vs can easily be found:

vs*i=150mW

vs=94.9V

v2=vs6=15.8V

The equivalent resistance seen by the voltage source is defined as the ratio of source voltage to the source current.

Req≡vsi=6R2=60kΩ

For resistors in series the equivalent resistance is:

Req=R1+R+R2+R3=60k

Thus:

R=Req-(R1+R2+R3)=30kΩ

Problem 2.33

A GE SoftWhite Longlife light bulb is rated as follows:

PR = rated power = 60 W

POR = rated optical power = 820 lumens (lm) (average)

1 lumen = 1/680W

Operating life = 1,500 h (average)

VR = rated operating voltage = 115 V

The resistance of the filament of the bulb, measured with a standard multimeter, is 16.7 Ω. When the bulb is connected into a circuit and is operating at the rated values given above, determine:

a. The resistance of the filament.

b. The efficiency of the bulb.

## Solution:

### Known quantities:

Rated power; rated optical power; operating life; rated operating voltage; open-circuit resistance of the filament.

### Find:

- The resistance of the filament in operation

- The efficiency of the bulb.

### Analysis:

a)

OL:

b)

Efficiency is defined as the ratio of the useful power dissipated by or supplied by the load to the total power supplied by the source. In this case, the useful power supplied by the load is the optical power. From any handbook containing equivalent units: 680 lumens=1 W

Optical Power Out

efficiency .

Problem 2.34

An incandescent light bulb rated at 100 W will dissipate 100 W as heat and light when connected across a 110-V ideal voltage source. If three of these bulbs are connected in series across the same source, determine the power each bulb will dissipate.

## Solution:

### Known quantities:

Rated power; rated voltage of a light bulb.

### Find:

The power dissipated by a series of three light bulbs connected to the nominal voltage.

### Assumptions:

The resistance of each bulb doesn’t vary when connected in series.

### Analysis:

When connected in series, the voltage of the source will divide equally across the three bulbs. The voltage across each bulb will be 1/3 what it was when the bulbs were connected individually across the source. Power dissipated by a resistor is proportional to the voltage squared, so the power dissipated in each bulb when connected in series will be 1/9 what it was when the bulbs were connected individually, or 11.11 W:

Ohm’s Law:

Connected in series and assuming the resistance of each bulb remains the same as when connected individually:

KVL:     OL:

Problem 2.35

An incandescent light bulb rated at 60 W will dissipate 60 W as heat and light when connected across a 110-V ideal voltage source. A 100-W bulb will dissipate 100 W when connected across the same source. If the bulbs are connected in series across the same source, determine the power that either one of the two bulbs will dissipate.

## Solution:

### Known quantities:

Rated power and rated voltage of the two light bulbs.

### Find:

The power dissipated by the series of the two light bulbs.

### Assumptions:

The resistance of each bulb doesn’t vary when connected in series.

### Analysis:

When the bulbs are connected in series, the voltage of the source will be divided across the two bulbs in proportion to the resistance of each bulb. The resistance of each bulb can be calculated using the definition of power and Ohm’s law.

Assume the resistance of each bulb is independent of temperature such that the resistance of each bulb when powered in series is the same as that of each bulb when powered individually. (In practice, the resistance of an incandescent bulb increases with temperature.) When powered in series:

KVL:

Solve for the current I.

Then:

Notes:

1. It may seem strange that when the bulbs are connected in series the 60 W bulb dissipates more power than the 100 W bulb. The reason is that the higher resistance of the 60W bulb results in greater voltage across it. Since the power dissipated is proportional to voltage squared the 60W bulb dissipates more power.

2. If the voltage across the filament of an incandescent bulb is decreased, the power it dissipates is also decreased, and therefore the temperature at which the filament operates is decreased. Since the resistance of the filament is an increasing function of temperature, the resistance of the bulb is decreased as well. This behavior necessitated the constant resistance assumption.

Problem 2.36

Refer to Figure P2.36, and assume that vS = 12 V, R1 =5 , R2 =3 , R3 =4 , and R4 =5 . Find:

- The voltage vab.

- The power dissipated in R2.

## Solution:

### Known quantities:

### Figure P2.36. R1=5Ω, R2=3Ω, R3=4Ω, R4=5Ω, vs=12V.

### Find:

vab and the power dissipated in R2.

### Analysis:

Apply voltage division to find the voltage across R2 and R4, respectively.
v2=vsR2R1+R2=4.5V

v4=vsR4R3+R4=203V

Apply KVL to find

vab=v2-v4=13.53-203=-6.53V≅-2.16V

The power dissipated by R2 is

PR2=v22R2=4.523=6.75W

Problem 2.37

For the circuit shown in Figure P2.37, find the currents

i2 and i1 and the power supplied by the voltage source.

## Solution:

### Known quantities:

### Circuit of Figure P2.37. R1=20Ω, R2=12Ω, R3=10Ω,

### is=3A, vs=7V.

### Find:

i1, i2, and Pv.

### Analysis:

Apply KVL to the left loop.

vs-R1*i1-R2*i2-R3*i1=0

vs=R1*i1+R2*i2+R3*i1

Apply KCL to the upper right node.

i1+is-i2=0

i1+is=i2

Combine the two equations to solve for i1.

vs=R1*i1+R2*(i1+is)+R3*i1

i1=-0.69A

Solve for i2.

i1+is=i2

i2=2.31A

Power delivered equals the vstimes i1:

Pv=vs*i1=-4.83W

Problem 2.38

For the circuit shown in Figure P2.38, find the currents i2 and i1 and the power supplied by the voltage sources.

## Solution:

### Known quantities:

### Circuit of Figure P2.38. R1=18Ω, R2=10Ω, v1=15V, v2=6V.

### Find:

i1, i2, Pv1,Pv2.

### Analysis:

Apply KVL to the right loop.

v2+R1*(-i1)-R2*i2=0

v2=R1*i1+R2*i2

Apply KCL to the upper node.

i-i1-i2=0

i1+i2=i

Note that the voltage across R2 is v1 such that

v1=R2*i2

i2=1.5A

Apply KVL around the left loop.

v1-v2=R1*i1

i1=0.5A

Solve for i.

i1+i2=i

i=2.0A

Power delivered equals the voltage source times the current through it:

P1=-v1*i=-30.0W=30.0W supplied

P2=v2*i1=3W=3.0W consumed

Problem 2.39

Consider NiMH hobbyist batteries shown in the circuit of Figure P2.39.

a. If V1 = 12.0 V, R1 = 0.15 Ω and R0 = 2.55Ω find the load current Io and the power dissipated by the load.

b. If we connect a second battery in parallel with battery 1 that has voltage V2 = 12 V and R2 = 0.28 Ω, will the load current Io increase or decrease? Will the power dissipated by the load increase or decrease? By how much?

## Solution:

### Known quantities:

Schematic of the circuit in Figure P2.39.

### Find:

- If , the load current and the power dissipated by the load

- If a second battery is connected in parallel with battery 1 with , determine the variations in the load current and in the power dissipated by the load due to the parallel connection with a second battery.

### Analysis:

a)	= 4.44 A

= 50.4 W.

b) Apply KVL twice using mesh currents to obtain 2 equations in 2 unknowns:

Solving the above equations gives us:

A,   A  A		W

This is an increase of 1%.

Problem 2.40

With no load attached, the voltage at the terminals of a particular power supply is 50.8 V. When a 10-W load is attached, the voltage drops to 49 V.

a. Determine vS and RS for this non-ideal source.

b. What voltage would be measured at the terminals in the presence of a 15-Ω load resistor?

c. How much current could be drawn from this power supply under short-circuit conditions?

## Solution:

### Known quantities:

Open-circuit voltage at the terminals of the power source is 50.8 V; voltage drop with a 10-W load attached is 49 V.

### Find:

- The voltage and the internal resistance of the source

- The voltage at its terminals with a 15- load resistor attached

- The current that can be derived from the source under short-circuit conditions.

### Analysis:

(a)	, the open-circuit voltage is

(b)

(c)	 A

Problem 2.41

A 220-V electric heater has two heating coils, which can be switched such that either coil can be used independently or the two can be connected in series or parallel, yielding a total of four possible configurations. If the warmest setting corresponds to 2,000-W power dissipation and the coolest corresponds to 300 W, find the resistance of each coil.

## Solution:

### Known quantities:

Voltage of the heater, maximum and minimum power dissipation; number of coils, schematics of the configurations.

### Find:

The resistance of each coil

### Analysis:

For the parallel connection,  W. Therefore,

or,

.

For the series connection,  W. Therefore,  or, .

Solving, we find that and .

# Problem 2.42

For the circuits of Figure P2.42, determine the resistor values (including the power rating) necessary to achieve the indicated voltages. Resistors are available in 1/8-, 1/4-, 1/2-, and 1-W ratings.

## Solution:

### Known quantities:

Circuits of Figure 2.42.

### Find:

Values of resistance and power rating

### Analysis:

(a)

(b)

c)

Problem 2.43

At an engineering site, a 1-hp motor is placed a distance d from a portable generator, as depicted in Figure P2.43. The generator can be modeled as an ideal DC source VG = 110 V. The nameplate on the motor gives the following rated voltages and full-load currents:

VMmin =105V→ IM FL =7.10A

VMmax =117V→ IM FL =6.37A

If d = 150 m and the motor must deliver its full-rated power, determine the minimum AWG conductors that must be used in a rubber-insulated cable. Assume that losses occur only in the wires.

## Solution:

### Known quantities:

Figure P2.43.

### Find:

The minimum AWG wire that will meet the specifications.

### Assumptions:

Assume an ideal DC source.

### Analysis:

The circuit as described is essentially two voltage sources in series with two resistors. The resistances in Table 2.1 are given in units of ohms per 1000 feet. The total length of wire is 150m or roughly 492 feet.

Assume the voltage of the motor is 105V. Use Ohm’s Law to calculate the Current through the first conductor:

IM=110V-105V2R

For the given motor rating, IM has to be 7.1 A for the motor to draw full power.

7.1A=2.5VR

R=0.352Ω

Use the value of R along with the chart to find the correct AWG:

0.352 492 ft=0.715 mΩft

In table 2.1, 8 AWG wire is rated at 0.64 m/ft. The AWG scale is logarithmic (similar to the decibel scale) such that adding 3 doubles the resistance. Since 6 AWG is 0.4 m/ft, 9 AWG is 0.8 m/ft. The desired rating is between these two values. It is important in practice that the wire gauge meets or exceeds the minimum specified value. Thus, a good choice would be the thicker of the two wire sizes, which is the 8 AWG wire.

Problem 2. 44

Cheap resistors are fabricated by depositing a thin layer of carbon onto a non-conducting cylindrical substrate (see Figure P2.44). If such a cylinder has radius a and length d, determine the thickness of the film required for a resistance R if

a = 1 mm 	R = 33 kΩ	σ = 1/ρ= 2.9 MS/m	d = 9 mm

Neglect the end surfaces of the cylinder and assume that the thickness is much smaller than the radius.

## Solution:

### Known quantities:

Figure P2.44.  Diameter of the cylindrical substrate; length of the substrate; conductivity of the carbon.

### Find:

The thickness of the carbon film required for a resistance R of 33 k.

### Assumptions:

Assume the thickness of the film to be much smaller than the radius

Neglect the end surface of the cylinder.

### Analysis:

Problem 2.45

The resistive elements of fuses, lightbulbs, heaters, etc., are significantly nonlinear (i.e., the resistance is dependent on the current through the element). Assume the resistance of a fuse (Figure P2.45) is given by the expression R = R0[1 + A(T −T0)] with T −T0 = kP; T0 = 25◦C; A = 0.7[◦C]−1; k = 0.35◦C/W; R0 = 0.11 Ω; and P is the power dissipated in the resistive element of the fuse. Determine the rated current at which the circuit will melt and open, that is, “blow” (Hint: The fuse blows when R becomes infinite.)

## Solution:

### Known quantities:

Figure P2.45.  The constants A and k; the open-circuit resistance.

### Find:

The rated current at which the fuse blows, showing that this happens at:

.

### Assumptions:

Here the resistance of the fuse is given by:

where , room temperature, is assumed to be 25°C.

We assume that:

where P is the power dissipated by the resistor (fuse).

### Analysis:

when

=6.09 A.

Problem 2.46

Use Kirchhoff’s current law and Ohm’s law to determine the current in each of the resistors R4, R5, and R6 in the circuit of Figure P2.46. VS = 10 V, R1 = 20 Ω, R2 = 40 Ω, R3 = 10 Ω, R4 = R5 = R6 = 15 Ω.

## Solution:

### Known quantities:

Circuit shown in Figure P2.46 with voltage source, and resistors, .

### Find:

The current in the 15- resistors.

### Analysis:

Since each of the 3 resistors in parallel have the same resistance they also have the same currents.

and,

mA

Therefore,  mA

Problem 2.47

Use Kirchhoff’s current law and Ohm’s law to determine the unknown currents.

## Solution:

### Known quantities:

Circuit shown in Figure P2.13

R0=1, R1=2, R2=3, R3=4, vs=10V

### Find:

a) i0 b) i1 c) i2 d) i3

### Analysis:

Use KVL at mesh A:

vs-i0∙R0-i1∙R1=0

10=i0∙1+i1∙2 Eq. 1

Use KVL at middle + right mesh:

i1∙R1=-i2∙R2-(i3∙R3) Eq. 2

Use KCL at node A:

i0+i2-i1=0

i1=i0+i2  Eq. 3

Use KCL at node B:

i3+6A-i2=0

i2=i3-6A Eq. 4

Use algebra to solve the four equations with four unknowns:

i0=1.83A

i1=4.09A

i2=2.26A

i3=-3.74A

Problem 2.48

Use Kirchhoff’s current law and Ohm’s law to determine the power delivered by the voltage source.

## Solution:

### Known quantities:

Circuit shown in Figure P2.48

k=0.25A/A2

### Find:

Power supplied by the voltage source.

### Analysis:

The 7 Ohm and 5 Ohm resistor are in series so i can be found by using Ohm’s law:

24V=12*i

i=2A

The dependent current source current can therefore be found using i:

id=0.25*(2A)2

id=1A

Use KCL to determine the current through the voltage source:

iv+i-id=0

iv=-1A

Use P=v*I to determine the power supplied by the voltage source:

P=24V*-1A

P=-24W=24W supplied

Problem 2.49

Assuming R0 = 2 Ω, R1 = 1Ω, R2 = 4/3 Ω R3 = 6 Ω and VS = 12 V in the circuit of Figure

P2.49, use Kirchhoff’s voltage law and Ohm’s law to find a. ia ,ib, and ic.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.49 with resistors  and voltage source V.

### Find:

- The mesh currents , ,

- The current through each resistor.

### Analysis:

Applying KVL to mesh (a), mesh (b) and mesh (c):

Solving the system we obtain:

Problem 2.50

Assuming R0 = 2Ω, R1 = 2 Ω, R2 = 5 Ω , R3 = 4 Ω, and VS = 24 V in the circuit of Figure P2.49, use Kirchhoff’s voltage law and Ohm’s law to find

a. ia, ib, and ic.

b. The voltage across each resistance.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.55 with resistors  and voltage source V.

### Find:

- The mesh currents , ,

- The current through each resistor.

### Analysis:

Applying KVL to mesh (a), mesh (b) and mesh (c):

Solving the system we obtain:

Problem 2.51

Assume that the voltage source in the circuit of Figure P2.49 is now replaced by a current source, and R0 = 1 Ω, R1 = 3 Ω, R2= 2Ω, R3 = 4 Ω, and IS = 12 A. Use Kirchhoff’s voltage law and Ohm’s law to determine the voltage across each resistance.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.49 with resistors R0 = 1 Ω, R1 = 3 Ω, R2= 2Ω, R3 = 4 Ω, and IS = 12 A.

### Find:

The voltage across each resistance.

### Analysis:

Applying KVL to mesh (a), mesh (b) and mesh (c):

Solving the system we obtain:

Problem 2.52

The voltage divider network of Figure P2.52 is expected to provide 5 V at the output. The resistors, however, may not be exactly the same; that is, their tolerances are such that the resistances may not be exactly 5 kΩ.

a. If the resistors have 10 percent tolerance, find the worst-case output voltages.

b. Find these voltages for tolerances of 5 percent.

Given: v = 10 V, R1 = 5 kΩ, R2 = 5 kΩ.

## Solution:

### Known quantities:

Schematic of voltage divider network shown of Figure P2.52.

### Find:

- The worst-case output voltages for  percent tolerance

- The worst-case output voltages for  percent tolerance

### Analysis:

a) 10% worst case: low voltage

R2 = 4500 , R1 = 5500

10% worst case: high voltage

R2 = 5500 , R1 = 4500

b) 5% worst case: low voltage

R2 = 4750 , R1 = 5250

5% worst case: high voltage

R2 = 5250 , R1 = 4750

Problem 2.53

Find the equivalent resistance seen by the source and the currents i and i1 and v2in the circuit of Figure P2.53.

## Solution:

### Known quantities:

Circuits of Figure 2.53.

### Find:

Equivalent resistance and current I and i1and v2

### Analysis:

Simplify resistors:

(9Ω|72Ω+2Ω=Req

Req=10Ω

Find i:

i=9VReq=9V10Ω

i=0.9A

Find i1 using current division:

i1=0.9A*72Ω(72Ω+9Ω)

i1=0.8A

Find voltage drop across 2 resistor which equals v2:

0.9A=9V-v22Ω

v2=7.2V

Problem 2.54

Find the equivalent resistance seen by the source

and the current i in the circuit of Figure P2.54.

## Solution:

### Known quantities:

Circuits of Figure 2.54.

### Find:

Equivalent resistance and current

### Analysis:

Step1:

Step 2:

Step 3: 4Ω+6Ω||90Ω=9Ω

The equivalent resistance seen by the voltage source is 10 and the voltage across the 9 resistance is 45V.

Thus, the current is easily calculated from Ohm’s law:

i=45V90Ω=0.5A

Problem 2.55

In the circuit of Figure P2.55, the power absorbed by the 15-Ω resistor is 15 W. Find R.

## Solution:

### Known quantities:

Circuits of Figure 2.55.

### Find:

Resistance R.

### Analysis:

Combining the elements to the right of the 15  resistor, we compute

.

The power dissipated by the 15- resistor is

,

Thus

v=15V, i1=1.0A, and i2=1.5A

Apply KCL to find

iR=i1+i2=2.5A

Apply KVL to find

25V=iRR+15V

Therefore, R = 4.

Problem 2.56

Find the equivalent resistance between terminals a and b in the circuit of Figure P2.56.

## Solution:

### Known quantities:

Network in Figure 2.56.

### Find:

Equivalent resistance.

### Analysis:

Problem 2.57

For the circuit shown in Figure P2.57, find the equivalent resistance seen by the source. How much power is delivered by the source?

## Solution:

### Known quantities:

Circuits of Figure 2.57.

### Find:

- Equivalent resistance

- Power delivered.

### Analysis:

(a)

(b)

P=14V210.8Ω=18.1W

Problem 2.58

For the circuit shown in Figure P2.58, find the equivalent resistance seen by the current source and the number of nodes.

## Solution:

### Known quantities:

Circuits of Figure 2.58.

### Find:

The equivalent resistance seen by the current source.

The number of nodes in the circuit.

### Analysis:

Simplify left side:

R1||R2+R3=Rleft

Rleft=86.2Ω

Simplify right side:

R4+R5=Rright

Rright=6Ω

Combine:

Req=Rleft||Rright

Req=5.61Ω

Count the nodes:

# of Nodes=4

Problem 2.59

For the circuit shown in Figure P2.59, find the number of nodes, the equivalent resistance seen by the voltage source, and the current through R7.

## Solution:

### Known quantities:

### Circuit of Figure 2.59. R1=10Ω, R2=5 Ω, R3=8 Ω,

### R4=2 Ω, R5=4 Ω, R6=2 Ω, R7=1 Ω, R8=10 Ω, vs=20V.

### Find:

The equivalent resistance seen by the voltage source.

The current through R7.

### Analysis:

Count the nodes:

# of Nodes=4

Combine R1 and R2:

R1||R2=3.33Ω

R4-6 are in parallel and in series with R1||2:

R4R5R6+R1||2=4.13Ω

The previous resistance is in parallel with R3:

R3||4.13Ω=2.73Ω

R7 is in parallel with R8:

R8||R7=0.91Ω

These last two resistances are in series and are equal to the equivalent resistance:

Req=3.63Ω

The current through R7||8 will be the same as the current through the independent voltage source. Thus

I=vsReq=5.50A

Problem 2.60

For the circuit shown in Figure P2.60, find the equivalent resistance seen by the voltage source and count the number of nodes in the circuit.

## Solution:

### Known quantities:

Circuit of Figure 2.60.
R1=12Ω, R2=5Ω, R3=8Ω, R4=2Ω, R5=4Ω, R6=2Ω, R7=1Ω, R8=10Ω, R9=10Ω

### Find:

The equivalent resistance seen by the voltage source.

The number of nodes in the circuit.

### Analysis:

Simplify starting on the right side:

R6||(R2+R7)=1.5 Ω

Simplify middle:

1.5 Ω+R8||R5=2.97 Ω

Finish with the left:

[((2.97+R9)R4+R1]|R3=5.05 Ω

Count the nodes:

# of Nodes=6

Problem 2.61

For the circuit shown in Figure P2.61, find the number of nodes in the circuit, the power delivered by the voltage source and the equivalent resistance seen by the voltage source.

## Solution:

### Known quantities:

Circuit of Figure 2.61. R1=9Ω, R2=4Ω, R3=4Ω, R4=5Ω, R5=4Ω.

### Find:

The number of nodes in the circuit.

The power delivered by the voltage source.

The equivalent resistance seen by the voltage source.

### Analysis:

Count the nodes:

Nodes=4

Simplify starting on the right side:

R5||R4=209

This result is in series with R3.

209+4=569Ω

Further, this result is in parallel with R2.

5694929=22492≅2.43 Ω

Finally, this result is in series with R1. Thus:

Req=2.43+9=11.43Ω

Find power delivered using ohms law:

vs=i*Req

i=vsReq

i=0.87A

P=vs*i

P=8.75W

Problem 2.62

Determine the equivalent resistance of the infinite

network of resistors in the circuit of Figure P2.62.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.62.

### Find:

The equivalent resistance  of the infinite network of resistors.

### Analysis:

We can see the infinite network of resistors as the equivalent to the circuit in the picture:

Therefore,

Problem 2.63

For the circuit shown in Figure P2.63, find the number of nodes in the circuit, the power delivered by the current source and the equivalent resistance seen by the current source.

## Solution:

### Known quantities:

Circuit of Figure 2.63.

### Find:

The number of nodes in the circuit.

The power delivered by the current source.

The equivalent resistance seen by the current source.

### Analysis:

Count the nodes:

Nodes=4

Simplify the right side:

R5+R4=Rright

Rright=6Ω

Simplify the left side:

R1||R2+R3=Rleft

Rleft=4.12Ω

Combine both sides:

Rright||Rleft=Req

Req=2.44Ω

Find power delivered using ohms law:

v=is*Req

v=12.21V

P=v*is

P=61.05W

Problem 2.64

In the circuit of Figure P2.64, find the equivalent resistance looking in at terminals a and b if terminals c and d are open and again if terminals c and d are shorted together. Also, find the equivalent resistance looking in at terminals c and d if terminals a and b are open and if terminals a and b are shorted together.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.64.

### Find:

The equivalent resistance at terminals a,b in the case that terminals c,d are a) open b) shorted; the same for terminals c,d with respect to terminals a,b.

### Analysis:

With terminals c-d open, Req= ,

with terminals c-d shorted, Req= ,

with terminals a-b open, Req= ,

with terminals a-b shorted, Req= .

Problem 2.65

In the circuit of Figure P2.64, find the equivalent resistance looking in at terminals a and b if terminals c is shorted to terminal a and terminal d is shorted to terminal b.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.64.

### Find:

The equivalent resistance seen by terminals a and b.

### Analysis:

Shorting a to c eliminates the 360Ω resistor. Shorting b to d eliminates the rightmost 540Ω resistor. This leaves the remaining two resistors in parallel with each other.

540Ω||180Ω=Req

Req=135Ω

Problem 2.66

For the circuit shown in Figure P2.66, find the number of nodes in the circuit and the equivalent resistance seen by the voltage source.

## Solution:

### Known quantities:

Circuit of Figure 2.66.

### Find:

The equivalent resistance seen by the voltage source.

The number of nodes in the circuit.

### Analysis:

Count the nodes:

Nodes=4

Simplify R2, R3, and R5 using a wye-delta transformation:

RAC=R2*R3+R2*R5+R3*R5R3=928=11.5 Ω

RAB=R2*R3+R2*R5+R3*R5R5=924=23.0 Ω

RBC=R2*R3+R2*R5+R3*R5R2=925=18.4 Ω

Simplify the resulting circuit:

Req=[(R1RAB+(R4RBC]||(RAC)

Req=5.26 Ω

Problem 2.67

Determine the voltage between nodes A and B in the circuit shown in Figure P2.67.

VS = 12 V

R1 = 11 kΩ	R3 = 6.8 kΩ

R2 = 220 kΩ	R4 = 0.22 MΩ

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.67 with source voltage, ; and resistances,

### Find:

The voltage between nodes A and B.

### Analysis:

Consider the node attached to the low potential side of the voltage source to be the reference node.  Also, note that R1 and R3 are in series as are R2 and R4.

By voltage division and KVL:

The voltage is negative indicating that node B is at a higher potential than node A.

Problem 2.68

For the circuit shown in Figure P2.68, find the number of nodes in the circuit, the power delivered by the voltage source and the equivalent resistance seen by the voltage source.

## Solution:

### Known quantities:

Circuit of Figure 2.68.

vS=15V, R1=12Ω, R2=5Ω, R3=8Ω, R4=2Ω, R5=4Ω, R6=2Ω, R7=1Ω, R8=10Ω, R9=10Ω

### Find:

The number of nodes in the circuit.

The power delivered by the voltage source.

The equivalent resistance seen by the voltage source.

### Analysis:

Mark the nodes one at a time to find a total of 6 nodes. R9 sees the same node at both of its terminals. It is, so to speak, in parallel with the short-circuit wire to its right and so has no impact on the equivalent resistance seen by the voltage source. Start at the right and work left.

R8||(R6+R7)=2.3 Ω

(2.3+R4)||R5=2.1 Ω

2.1 Ω+(R3||R1+R2)=7.51Ω=Req

The power delivered by the voltage source is

P=vS2Req=29.9 W

Problem 2.69

For the circuit shown in Figure P2.69, find the voltages vac and vbd. Also find the power delivered by the voltage source.

## Solution:

### Known quantities:

Circuit of Figure 2.69. R1=12Ω, R2=5Ω, R3=8Ω, R4=2Ω,

R5=4Ω, R6=2Ω, R7=1Ω,vs=15V.

### Find:

vac, vbd, and the power delivered by the voltage source.

### Analysis:

Find Req seen by the voltage source by adding all the resistors in series:

Req=R1+R2+R3+R4+R5+R6+R7

Req=34 Ω

Use the equivalent resistance and voltage division to find vac and vbd.

vac=R1+R2+R3ReqvS=25 Ω34 Ω15 V=11.0 V

vbd=15 V-R1+R7ReqvS=15 V-13 Ω34 Ω15 V=9.3 V

The power delivered by the voltage source is

P=vS2Req=15234=6.6 W

# Problem 2.70

A thermistor is a nonlinear device which changes its terminal resistance value as its surrounding temperature changes. The resistance and temperature generally have a relation in the form of Rth(T ) = R0e−β(T−T0), where Rth = resistance at temperature T, (Ω), R0 = resistance at temperature T0 = 298 K, (Ω), β = material constant, (K−1), T, T0 = absolute temperature, (K).

a. If R0 = 300Ω and β = −0.01 K−1, plot Rth as a function of the surrounding temperature T for 350 ≤ T ≤ 750.

b. If the thermistor is in parallel with a 250-Ω resistor, find the expression for the equivalent resistance and plot Rth(T ) on the same graph for part a.

## Solution:

### Known quantities:

Parameters  (resistance at temperature = 298 K), and, value of the second resistor.

### Find:

- Plot  versus T in the range [K]

- The equivalent resistance of the parallel connection with the 250- resistor; plot  versus T in the range [K] for this case on the same plot as part a.

### Assumptions:

.

### Analysis:

a)

b)

The two plots are shown below.

In the above plot, the solid line is for the thermistor alone; the dashed line is for the thermistor-resistor combination.

Problem 2.71

A moving-coil meter movement has a meter resistance rM= 200 Ω, and full-scale deflection is caused by a meter current Im= 10 μA. The movement must be used to indicate pressure measured by the sensor up to a maximum of 100 kPa. See Figure P2.71.

a. Draw a circuit required to do this, showing all appropriate connections between the terminals of the sensor and meter movement.

b. Determine the value of each component in the circuit.

c. What is the linear range, that is, the minimum and maximum pressure that can accurately be measured?

## Solution:

### Known quantities:

Meter resistance of the coil; meter current for full scale deflection; max measurable pressure.

### Find:

- The circuit required to indicate the pressure measured by a sensor

- The value of each component of the circuit; the linear range

- The maximum pressure that can accurately be measured.

### Assumptions:

Sensor characteristics follow what is shown in Figure P2.71

### Analysis:

a) A series resistor to drop excess voltage is required.

b) At full scale, meter:

at full scale, sensor (from characteristics):

Ohm law: .

c) from sensor characteristic: 30 kPa –110 kPa.

Problem 2.72

The circuit of Figure P2.72 is used to measure the internal impedance of a battery. The battery being tested is a NiMH battery cell.

a. A fresh battery is being tested, and it is found that the voltage Vout , is 2.28 V with the switch open and 2.27 V with the switch closed. Find the internal resistance of the battery.

b. The same battery is tested one year later, and Vout is found to be 2.2 V with the switch open but 0.31 V with the switch closed. Find the internal resistance of the battery.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.72; voltage at terminals with switch open and closed for fresh battery; same voltages for the same battery after 1 year.

### Find:

The internal resistance of the battery in each case.

### Analysis:

a)

b)

Problem 2.73

Consider the practical ammeter, described in Figure P2.73, consisting of an ideal ammeter in series with a 1-kΩ resistor. The meter sees a full-scale deflection when the current through it is 30 μA. If we desire to construct a multirange ammeter reading full-scale values of 10 mA, 100 mA, and 1 A, depending on the setting of a rotary switch, determine appropriate values of R1, R2, and R3.

## Solution:

### Known quantities:

Ammeter shown in Figure P2.73; Current for full-scale deflection; desired full scale values.

### Find:

Value of the resistors required for the given full scale ranges.

### Analysis:

We desire R1,R2, R3 such that Ia= 30 A for I = 10 mA, 100 mA, and 1 A, respectively. We use conductances to simplify the arithmetic:

Ga = =S

G1,2,3 =

By the current divider rule:

Ia = I

or:

or

.

We can construct the following table:

| x | I | Rx (Approx.) |
| --- | --- | --- |
| 1 | 10-2 A | 3 |
| 2 | 10-1 A | 0.3 |
| 3 | 100 A | 0.03 |

Problem 2.74

A circuit that measures the internal resistance of a practical ammeter is shown in Figure P2.74, where RS = 50,000 Ω, vS = 12 V, and Rp is a variable resistor that can be adjusted at will.

a. Assume that ra << 50,000 Ω. Estimate the current i.

b. If the meter displays a current of 150 μA when Rp = 15 Ω, find the internal resistance of the meter ra.

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.74; for part b: value of  and current displayed on the ammeter.

### Find:

The current i; the internal resistance of the meter.

### Assumptions:

### Analysis:

a) Assuming that

i = 240 A

b) With the same assumption as in part a)

= 150(10)-6 =

or:

150(10)-6= .

Therefore, = 9 .

Problem 2.75

A practical voltmeter has an internal resistance rm. What is the value of rm if the meter reads 11.81V when connected as shown in Figure P2.75.

## Solution:

### Known quantities:

Voltage read at the meter; schematic of the circuit shown in Figure P2.75 with source voltage, and source resistance,.

### Find:

The internal resistance of the voltmeter.

### Analysis:

Using the voltage divider rule:

V = 11.81 =

Therefore, = 1.55 M.

Problem 2.76

Using the circuit of Figure P2.75, find the voltage that the meter reads if VS = 24 V and RS has the following values: RS = 0.2rm, 0.4rm, 0.6rm, 1.2rm, 4rm, 6rm, and 10rm. How large (or small) should the internal resistance of the meter be relative to RS?

## Solution:

### Known quantities:

Circuit shown in Figure P2.75 with source voltage, ; and ratios between  and .

### Find:

The meter reads in the various cases.

### Analysis:

By voltage division:

V =

| Rs | V |
| --- | --- |
| 0.2 | 20 V |
| 0.4 | 17.14 V |
| 0.6 | 15 V |
| 1.2 | 10.91 V |
| 4 | 4.8 V |
| 6 | 3.43 V |
| 10 | 2.18 V |

For a voltmeter, we always desire  >> .

Problem 2.77

A voltmeter is used to determine the voltage across a resistive element in the circuit of Figure P2.77. The instrument is modeled by an ideal voltmeter in parallel with a 120-kΩ resistor, as shown. The meter is placed to measure the voltage across R4. Assume R1 = 8 kΩ, R2 = 22 kΩ, R3 = 50 kΩ, RS = 125 kΩ, and IS = 120 mA. Find the voltage across R4 with and without the voltmeter in the circuit for the following values:

a. R4 = 100 Ω

b. R4 = 1 kΩ

c. R4 = 10 kΩ

d. R4 = 100 kΩ

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.77, values of the components.

### Find:

The voltage across  with and without the voltmeter for the following values:

a)

b)

c)

d)   .

### Assumptions:

The voltmeter behavior is modeled as that of an ideal voltmeter in parallel with a 120- k resistor.

### Analysis:

We develop first an expression for  in terms of . Next, using current division:

Therefore,

Without the voltmeter:

a) = 3.08 V

b) = 30.47 V

c) = 269.91 V

d) = 1260.7 V.

Now we must find the voltage drop across  with a 120-k resistor across . This is the voltage that the voltmeter will read.

With the voltmeter:

a) = 3.08 V		b) = 30.47 V

c) = 272.57 V	d) = 1724.99 V.

Problem 2.78

An ammeter is used as shown in Figure P2.78. The ammeter model consists of an ideal ammeter in series with a resistance. The ammeter model is placed in the branch as shown in the figure. Find the current through R5 with and without the ammeter in the circuit for the following values, assuming that RS = 20 Ω, R1 =800 Ω, R2 = 600 Ω, R3 = 1.2 kΩ, R4 = 150 Ω, and VS = 24 V.

a. R5 = 1 kΩ

b. R5 = 100 Ω

c. R5 = 10 Ω

d. R5 = 1 Ω

## Solution:

### Known quantities:

Schematic of the circuit shown in Figure P2.78 and the values of the components.

### Find:

The current through  with and without the ammeter, for the following values of :

a)

b)

c)

d)  .

### Analysis:

First we should find an expression for the current through  in terms of  and the meter resistance, . By the voltage divider rule we have:

=

And	 =

Therefore,

=

Using the above equation will give us the following table:

|  | with meter in circuit | without meter in circuit |
| --- | --- | --- |
| a | 10.15 mA | 9.99 mA |
| b | 24.03 mA | 23.15 mA |
| c | 27.84 mA | 26.67 mA |
| d | 28.29 mA | 27.08 mA |

Problem 2.79

Shown in Figure P2.79 is an aluminum cantilevered beam loaded by the force F. Strain gauges R1, R2, R3, and R4 are attached to the beam as shown in Figure P2.79 and connected into the circuit shown. The force causes a tension stress on the top of the beam that causes the length (and therefore the resistance) of R1 and R4 to increase and a compression stress on the bottom of the beam that causes the length (and therefore the resistance) of R2 and R3 to decrease. This causes a voltage of 50 mV at node B with respect to node A. Determine the force if Ro = 1 kΩ, VS = 12 V, L = 0.3 m, w = 25 mm, h = 100 mm, and Y = 69 GN/m2.

## Solution:

### Known quantities:

Schematic of the circuit and geometry of the beam shown in Figure P2.79, characteristics of the material, reads on the bridge.

### Find:

The force applied on the beam.

### Assumptions:

Gage Factor for strain gauge is 2.

### Analysis:

and  are in series;  and  are in series.

Voltage Division:

Voltage Division:

KVL:

assuming GF=2 for aluminum.

# Problem 2.80

Shown in Figure P2.80 is a structural steel cantilevered beam loaded by a force F. Strain gauges R1, R2, R3, and R4 are attached to the beam as shown and connected into the circuit shown. The force causes a tension stress on the top of the beam that causes the length (and therefore the resistance) of R1 and R4 to increase and a compression stress on the bottom of the beam that causes the length (and therefore the resistance) of R2 and R3 to decrease. This generates a voltage between nodes B and A. Determine this voltage if F = 1.3 MN, Ro = 1 kΩ, VS = 24 V L = 1.7 m, w = 3 cm, h = 7 cm, Y = 200 GN/m2

## Solution:

### Known quantities:

Schematic of the circuit and geometry of the beam shown in Figure P2.79, characteristics of the material, reads on the bridge.

### Find:

The force applied on the beam.

### Assumptions:

Gage Factor for strain gauge is 2.

### Analysis:

and  are in series;  and  are in series.

Voltage Division:

Voltage Division:

KVL:

Assuming GF=2 for aluminum.

E. O. Doebelin, Measurement Systems – Application and Design, 5th Edition, McGraw-Hill, New York, 2004.